A Wearable Respiratory Monitoring Device

Abstract

1.Introduction

- Motivation

- Problem Statement

- Work Contribution

- Outline of the Paper

- Literature Survey

Xubin Zheng et.al., [1] proposed a mask with a novel hydrophobic composite material, mCP/ZIF-8@COF, synthesized to create a CO₂ sensor that analyzes breathing patterns by monitoring exhaled carbon dioxide. A residual neural network (RNN) algorithm is used to improve COPD diagnostic accuracy. The limitations are that it is not suitable for long-term use due to the limited lifetime of the CO₂ sensor, and it relies only on end-tidal CO₂ levels, which can sometimes lead to misclassification.

Chandana H et.al., [2] proposed a Fiber Bragg Grating (FBG) – based wearable system integrated with a PVC oxygen mask for early detection of COVID-19 and acute respiratory syndrome. This system accurately tracks different breathing patterns based on strain-induced wavelength shifts. It is lightweight, EMI-resistant and suitable for environments like MRI. However, it is less portable due to its external optical hardware and not yet evaluated in different conditions like varied temperature, long-term wear and during movement.

Lin Gao et.al., [3] proposed a smart mask with a high-resolution pressure sensor based on melt-blown fibres with multipurpose conductive polymer PEG assisted PEDOT: PSS for respiratory monitoring. The system detects various breathing states including cough, exercise and sitting. It maintained comfort and mask functionality in real-time respiration monitoring. The limitation here is that its performance can be slightly affected by humidity.

K.Thiyagarajan et.al., [4] proposed a flexible paper-based respiration sensor embedded in a face mask.It uses    MWCNTs and PDMS on a paper substrate that measures respiration rate and senses humidity through  changes in dielectric constant.The sensor detects inhalation and exhalation by measuring capacitance variations during breathing.The limitation here is it lacks wireless capability, making it less portable for real-time mobile use and only respiration rate and humidity are measured.

Yu et.al., [5] proposes a smart mask that uses a flexible fiber optic senesor. It detects breath patterns, respiration rate. Sensor is affixed outside mask to avoid cross infection. It is characterized by ultra-thin, flexible, high       sensitivity, and fast rise time (0.19 s). The limitation here is that a hardware must be carried by the patient to transmit data for monitoring because fiber optic sensors can transfer data through optical fibers only.

Blanco–Almazán et.al., [6] Proposed a wearable device which should be attached to patients body.It detects respiration rate.The main objective of this study was to assess and compare the performance of two EDR methods, using nonstandard ECG leads.The limitation here is that continues monitoring is not possible as  skin start to irritate.

3.Proposed Methodology

4.Results & Analysis

5.Performance Evaluation

6.Conclusion

References